INFORMATION ON NEW CONCLUSIONS OF DOCTORAL DISSERTATION

(Information will be posted on the Website)

Name of dissertation: Performance enhancement solutions for selected cryptographic algorithms in IP network security.

Field of study: Cryptography Techniques                     Code No: 9520209

Name of Ph.D. Student: Trần Sỹ Nam

Advisor 1: Assoc. Prof. Dr. Lương Thế Dũng

Advisor 2: Dr. Hoàng Văn Thức

Training Institution: Academy Of Cryptography Techniques

Summary of new contributions of the Dissertation

1. A 4-stage pipelined AES-P architecture is proposed, incorporating new cryptographic components, namely S-box S_p and  MDS matrix M_p, designed with hardware-friendly structures. The proposed AES-P architecture achieves a throughput of 105.7 Gbps with a latency of 48.4 ns, reducing latency by up to 53% compared to related works on the same Virtex-7 FPGA platform. The security of AES-P is preserved against differential and linear cryptanalysis, while enhancing resistance to algebraic attacks, with consideration toward potential quantum computing threats. In addition, a novel architecture for Ascon is proposed, comprising both the cryptographic core and interface modules, achieving a throughput of 13.312 Gbps per cycle, eliminating memory dependency, and making it well-suited for real-time IoT systems..

2. The HMS-IPSec hardware architecture is designed and implemented, integrating AES-P-GCM, AES-P-CTR-HMAC-SHA256, and Ascon-AEAD128. The architecture features a strict separation between the data plane and the control plane, combined with full support for essential features such as NAT traversal and 64-bit Extended Sequence Numbers (ESN), thereby addressing practical deployment limitations of prior works. It enables flexible switching between AES-P and Ascon without modifying the packet processing structure, ensuring both high throughput and low latency for real-time applications. This is the first work to systematically evaluate the feasibility and effectiveness of integrating the lightweight cipher Ascon-AEAD128 into the IPsec protocol on a hardware SoC platform.

The research aims to optimize throughput, reduce latency, and minimize hardware overhead in IP network security, addressing the critical demands of 5G/6G systems and real-time applications.

Hanoi, December ……, 2026.

| Advisors (Signature) Assoc. Prof. Dr. Lương Thế Dũng Dr. Hoàng Văn Thức | Ph.D. Student (Signature) Trần Sỹ Nam |
| --- | --- |